We are relying on a lot of assumptions & fixed values. In terms of non-static data, we need:

| Dataset | Granularity | Exact Source to Download | Why Your Equations Require It |
| --- | --- | --- | --- |
| 1. Wholesale Power Prices | 15-minute | ERCOT Real-Time Market (RTM) Settlement Point Prices (SPP) for LZ_NORTH (CoServ’s load zone). Available as CSV on ERCOT’s Market Information or GridStatus.io. | Drives Base’s revenue in the objective function: (power price) × (power to base) × 0.25. Also feeds (price weight) · (power price)in CoServ's benefit metric. |
| 2. ERCOT Grid Load (Load_t) | Hourly or 15-minute | ERCOT Hourly System-Wide Load (Actual or Day-Ahead Forecast) from ERCOT Grid Info archives. | Calculates the 4CP stress ratio: SystemLoad_t / SystemPeak. Without this, the model cannot identify when summer afternoon peaks occur. |
| 3. Ambient Temperature | Hourly | NOAA / NWS Local Climatological Data or Open-Meteo API for North Texas (e.g., Dallas-Fort Worth / Denton airport station KDFW or KDTO). | Computes the temperature penalty in your benefit equation: max(0, Temp at t - Temp threshold), and evaluates your piecewise inverter thermal derating (max power output of inverter). |

Simplest Setup

| Variable | Source | Data to Pull |
| --- | --- | --- |
| Power price | ERCOT real-time SPP | LZ_NORTH, 15-min |
| Load | ERCOT real-time load | NORTH_C weather zone, 15-min if available |
| Temperature | Open-Meteo | Hourly temperature around Denton/CoServ |

1. ERCOT API — Free

ERCOT's public API is free; you need an account/subscription key, not a paid subscription. Register, subscribe to the public API product, and obtain a subscription key.

ERCOT API Explorer: https://developer.ercot.com/

For real-time prices, pull:

NP6-905-CD
settlementPoint = LZ_NORTH
settlementPointType = LZ

ERCOT publishes settlement-point prices every 15 minutes. CoServ does not have its own ERCOT price zone; LZ_NORTH is the appropriate ERCOT wholesale-price proxy for CoServ's North Texas service territory. CoServ serves parts of Denton, Collin, Cooke, Grayson, Tarrant, and Wise counties.

2. Real-Time Load

Use ERCOT's System-Wide Demand / Actual Loads data. ERCOT does not publicly provide a real-time load series specifically labeled 'CoServ load' because its public load data is reported by geographic weather zones.

For a CoServ-region proxy, use:

NORTH_C

The North Central weather zone is the most relevant geographic load proxy for the core Denton/Collin CoServ territory. If strictly 15-minute real-time load is required, use ERCOT's System-Wide Demand feed; geographic specificity is limited to the zones ERCOT publishes.

3. Temperature — Easiest Option

Use Open-Meteo. It is free, requires no API key or account, and provides current and hourly weather data.

Open-Meteo: https://open-meteo.com/

For CoServ, use Denton, TX as the representative location:

latitude: 33.2148
longitude: -97.1331

API endpoint:

https://api.open-meteo.com/v1/forecast?latitude=33.2148&longitude=-97.1331&current=temperature_2m&hourly=temperature_2m&temperature_unit=fahrenheit&timezone=America/Chicago

Final Data Pipeline

ERCOT API → LZ_NORTH price (15-min) + NORTH_C load (real-time) + Open-Meteo → Denton temperature (hourly) → Model dataset